Oligonucleotide 5’ adenylation

Using a 5’ phosphorylated oligonucleotide and the 5’ adenylation kit from NEB

Set up reaction:

Phosphorylated DNA	Oligonucleotide	5 µl (500 pmol)

10x 5´ DNA Adenylation Reaction Buffer	5 µl

1 mM ATP					5 µl

Nuclease-free Water				to 45 µl

Mth RNA Ligase				5 µl

Incubate at 65°C for 1 hour.

Incubate 85C for 5min

Lid at 95deg

Dilute to 100ul with water, extract with 100µl phenol:chloroform pH8

Add 10µl 3M NaOAc, 1µl glycogen and 250µl 100% ethanol

Vortex

Spin 15 min top speed at 4C

Wash pellet with 400µl 70% ethanol

Normal oligos – use this for TrAEL-seq adaptor

Dissolve pellet in 50ul 0.1xTE (to 10pM/µl final concentration)

Vortex then let sit at room temperature for 30min or so

Store at -30C

Hairpin adaptors

Dissolve pellet in 45ul 0.1xTE (to 10pM/µl final concentration)

Vortex then let sit at room temperature for a few minutes

Add 5µl 10x buffer

Use T4 DNA ligase buffer for ENDseq, NEBuffer 1 for LIG-seq

Place in a heating block at 95C, remove block from the heat and allow to cool to room temperature over a few hours

Store at -30C