Denaturing poly-acrylamide gel

The trick to getting a nice PAGE gel is that the gel should be hot and the heat distributed uniformly. This is exactly the opposite of what you want in a protein gel, and protein gel tanks are designed to dissipate heat. We routinely use old school Biorad minigel tanks that have a Perspex block held against the gel (that is what this protocol is written for). For larger gel tanks, or other styles, a ~1mm sheet of aluminium sandwiched against the outside of the gel works well to spread the heat. Make sure that this is high enough not to dip in the buffer and low enough so that you can see your samples as you load them.

These instructions are for a 1mm wide, 7cm acrylamide minigel:

Make up a 10ml stock of PAGE solution of the appropriate percentage:

Weigh 4.8g urea into a 15ml tube

Add 1ml 10x TBE

Add the required amount of 29:1 acrylamide from 30% stock

Add water to 10ml

Heat gently and swirl (use a 55 water bath) until urea dissolves

Top up to 10ml exactly with water

Clean glass plates with water then ethanol, assemble with 1mm spacers

Add to 10ml PAGE mix:

50l 10% APS, invert to mix

10l TEMED, invert to mix

Pour immediately and insert the comb (make sure this is 1mm)

Allow to set for 30min, then assemble the apparatus but leave the comb in

Meanwhile, mix samples 1:1 with PAGE loading dye

Denature samples 5 minutes at 95

Meanwhile, warm 500ml 1xTBE to ~55 (2 min in our microwave) and pour in tank

Snap chill samples on ice, spin briefly and keep on ice until loading

Pull the comb, clean wells with syringe and immediately load samples

Run at 300V (7cm minigel) (Really! – this keeps the gel hot), dye migration is given in the table below

| % acrylamide | Xylene cyanol (nt) | Bromophenol blue (nt) |
| --- | --- | --- |
| 5 | 130 | 35 |
| 6 | 106 | 29 |
| 8 | 76 | 26 |
| 10 | 55 | 12 |
| 20 | 28 | 8 |

Blotting:

Wash gel 2x 5 min with 0.5x TBE, first wash containing SYBR if image is needed.

Transfer to Hybond N+ using Invitrogen NOVEX transfer system, according to manufacturer’s instructions.

Band extraction:

Locate band by SYBR gold staining or autoradiography and excise with a blade

Wipe the handle of a plastic spreader with RNAseZap (Ambion)

Mash up gel splice in 1.5ml tube using handle of plastic spreader

Add 0.5ml PAGE elution buffer

Incubate at 37 with agitation for 1-3hrs

Spin buffer through Spin-X column to remove gel pieces

Phenol chloroform extract

Add 1ml EtOH, 1l glycogen and precipitate, wash, re-suspend

2x PAGE loading dye:	95% formamide

0.025% bromophenol blue

0.025% xylene cyanol

5mM EDTA

0.025% SDS

store long term at -20, up to 1 month at RT

PAGE elution buffer:	0.5M NaOAc

1mM EDTA

0.1% SDS